Renew.Energy Types&Mod.Tech.

INS4069E

CRN: 11383

Course instructors: Prof. Dr. Cihan BAYINDIR

Research Assistant Selahattin Utku Yılmaz

Research Assistant Sinan Eruçar

Prepared by : Sepehr Sadeghzadeh Saadat 010200912

Table of Contents

1.Introduction	3

2. Methodology	5

2.1 Data Acquisition and Site Selection	5

2.2 Theoretical Power Generation (Target Synthesis)	7

2.2.1 Vertical Extrapolation	7

2.2.2 The Cubic Power Curve Model	8

2.3 Machine Learning Models and Hyperparameters	8

2.4 Experimental Design and Case Studies	9

Case 1: Baseline Forecasting (Chicago)	9

Case 2: Input Sensitivity Analysis	9

Case 3: Geographical Transferability	9

2.6 Performance Metrics	9

3. Result and discussion	10

3.1 Case 1 Results (Chicago: Mean + Std Dev)	10

3.2 Case 2 Results (Chicago: Mean Only)	11

3.3 Case 3 Results (Transferability to Detroit)	12

4. Conclusion	13

5.References	14

# 1.Introduction

Wind energy is one of the most critical and promising resources for sustainable energy production among renewable sources. However, the intermittent and unpredictable nature of wind poses significant challenges for energy production planning. Demolli et al. (2019) proposed a comprehensive methodology for long-term wind power forecasting using daily wind speed data and demonstrated the effectiveness of machine learning algorithms such as Random Forest, SVR, and XGBoost in this field.

Based on the methodological framework proposed by Demolli et al., this report aims to take the geographical generalizability and technical performance of the model a step further. While the original study analyzed specific regions in Turkey, such as Nigde and Cesme, this research selects Chicago and Detroit—cities in the United States with high wind potential—to test the universality of the model. Chicago was designated as the 'base location' for model training and initial validation, while Detroit was utilized as an independent 'blind test' set to measure the transferability of the models to a different geographical climate.

The locations of the meteorological stations shown on the map.

Technically, this study modernizes the existing approaches in the literature in two fundamental ways:

- Algorithmic Expansion: In addition to the traditional regression algorithms used in the original paper, LightGBM, one of today's highest-performing gradient boosting methods, has been integrated into the system.

- Optimization Strategy: Instead of the trial-and-error method used in the original study, Bayesian hyperparameter optimization was performed using the Optuna library. This approach maximized the forecasting capacity of the models on the complex wind patterns of Chicago and Detroit.

The locations of the selected stations shown on the map.

# 2. Methodology

This study adopts and extends the methodological framework proposed by Demolli et al. [1] to forecast long-term wind power generation. While the original study focused on locations in Turkey, this research applies the methodology to a new geographical context (Chicago and Detroit, USA) to test the generalizability of the proposed machine learning models. Furthermore, this study introduces a modern algorithmic extension (LightGBM) and a robust Bayesian optimization strategy (Optuna) to improve upon the original trial-and-error approach.

## 2.1 Data Acquisition and Site Selection

Hourly meteorological data was obtained from the open-access "Historical Hourly Weather Data 2012-2017" dataset available on Kaggle [3]. The city of Chicago was selected as the primary "Base Location" for training and initial validation due to its consistent wind characteristics. Additionally, data for a second city, Detroit, was acquired to serve as an independent test set for evaluating the geographical transferability of the models.

The dataset spans exactly five years, from October 1, 2012, to October 1, 2017. Following the protocol of the original study, the first four years (80%) were utilized for training and validation, whil the final year (20%) was reserved strictly for testing.

Figure 2: Wind Speed Characteristics

Figure 3: Weibull Frequency Distribution

## 2.2 Theoretical Power Generation (Target Synthesis)

A critical challenge in wind energy forecasting is the lack of public datasets containing both meteorological inputs and actual turbine power output. To address this, the target variable (Power Output) was synthesized mathematically based on the physics of a specific wind turbine.

### 2.2.1 Vertical Extrapolation

The raw meteorological data provided wind speeds at a standard height of 10 meters, which corresponds to the measurement standards used by services such as the OpenWeatherMap API [4]. However, commercial wind turbines operate at significantly higher altitudes. Therefore, the wind speed was extrapolated to a hub height of 50 meters using the Power Law equation:

v=v0hh0α

Where v is the wind speed at hub height (50m), v0 is the measured speed at 10m, and alpha is the Hellman exponent (set to 0.14 for open terrain).

### 2.2.2 The Cubic Power Curve Model

In the reference study by Demolli et al., the power generation data was derived using specific manufacturer power/speed tables. In contrast, this study utilizes a theoretical approach to ensure replicability. The relationship between wind speed and power generation was modeled using the "Four-Region" logic described in the course lecture notes by Bayindir [2].

Specifically, for the operational range (Region 2), the power output is calculated using the cubic formula visualized in Figure 1:

PT(v)≈a∙v3-b∙PR

Figure 1: Theoretical Power Curve

This formula ensures that the power output follows the physical cubic law of kinetic energy. The turbine parameters were set to: Cut-in (3 m/s), Rated (15 m/s), and Cut-out (25 m/s).

## 2.3 Machine Learning Models and Hyperparameters

Six regression algorithms were implemented to model the relationship between wind statistics and power generation. To strictly validate the findings of the original study, the hyperparameters for the five benchmark models were kept identical to those reported by Demolli et al. [1]. No additional parameter optimization was performed for these specific models.

The fixed configuration for the benchmark models is as follows:

LASSO Regression: alpha=0.1

k-Nearest Neighbors (kNN): k=4, Metric=Minkowski

Random Forest (RF): n_estimators=10

XGBoost: n_estimators=500, learning_rate=0.1

Support Vector Regression (SVR): C=3000, gamma=0.1, epsilon=0.1, kernel=RBF

Algorithmic Extension (LightGBM): The sixth model, LightGBM, was not present in the original study. Its hyperparameters were optimized using Optuna, a Bayesian optimization framework.

## 2.4 Experimental Design and Case Studies

To comprehensively evaluate the models, the methodology was structured into three distinct experimental cases.

### Case 1: Baseline Forecasting (Chicago)

Objective: To establish the baseline performance of the algorithms using the complete feature set.

Inputs: Daily Mean Wind Speed + Daily Standard Deviation.

Methodology: Training and testing were performed on the Chicago dataset. This duplicates 'Case 1' from the reference paper.

### Case 2: Input Sensitivity Analysis

Objective: To quantify the impact of wind volatility (turbulence) on prediction accuracy.

Inputs: Daily Mean Wind Speed only.

Methodology: The 'Standard Deviation' feature was removed. This mirrors 'Case 2' of the original study.

### Case 3: Geographical Transferability

Objective: To assess the generalization capability of the models when applied to a new location.

Inputs: Daily Mean Wind Speed + Daily Standard Deviation.

Methodology: Models trained on Chicago were frozen and used to predict power generation in Detroit.

## 2.6 Performance Metrics

To quantitatively evaluate the forecasting accuracy, three standard statistical metrics were employed:

Root Mean Squared Error (RMSE):

RMSE=1Nyi-yi2

Mean Absolute Error (MAE):

MAE=1Nyi-yi

Coefficient of Determination (R²):

R2=1-yi-yi2yi-y2

# 3. Result and discussion

This section presents the performance of the proposed algorithms across the three experimental cases.

## 3.1 Case 1 Results (Chicago: Mean + Std Dev)

Table 1 summarizes the forecasting accuracy for Case 1. XGBoost and SVR achieved the highest performance.

Figure 4: Case 1 Time Series Forecast (SVR)

| Forecasting accuracy of the algorithms for daily wind speed and standard deviation (Case 1). |  |  |  |
| --- | --- | --- | --- |
| Algorithm | R2 | RMSE (kW) | MAE (kW) |
| LASSO | 0.8687 | 697.35 | 469.8 |
| kNN | 0.9872 | 217.87 | 86.07 |
| XGBoost | 0.995 | 135.6 | 57.56 |
| Random Forest | 0.9942 | 146.86 | 67.37 |
| SVR | 0.9968 | 108.65 | 36.03 |
| LightGBM | 0.9949 | 137.49 | 53.69 |

## 3.2 Case 2 Results (Chicago: Mean Only)

Table 2 presents the results when the standard deviation feature is removed.

Figure 5: Case 2 Scatter Plot (Wind Speed vs Power)

| Forecasting accuracy of the algorithms for daily wind speed and standard deviation (Case 1). |  |  |  |
| --- | --- | --- | --- |
| Algorithm | R2 | RMSE (kW) | MAE (kW) |
| LASSO | 0.8687 | 697.35 | 469.8 |
| kNN | 0.9872 | 217.87 | 86.07 |
| XGBoost | 0.995 | 135.6 | 57.56 |
| Random Forest | 0.9942 | 146.86 | 67.37 |
| SVR | 0.9968 | 108.65 | 36.03 |
| LightGBM | 0.9949 | 137.49 | 53.69 |

## 3.3 Case 3 Results (Transferability to Detroit)

Table 3 compares the transferability of the models when applied to the Detroit dataset (Blind Test).

Figure 6: Case 3 Transferability Forecast (Detroit)

| Forecasting accuracy of the algorithms for daily wind speed and standard deviation (Case 1). |  |  |  |
| --- | --- | --- | --- |
| Algorithm | R2 | RMSE (kW) | MAE (kW) |
| LASSO | 0.8687 | 697.35 | 469.8 |
| kNN | 0.9872 | 217.87 | 86.07 |
| XGBoost | 0.995 | 135.6 | 57.56 |
| Random Forest | 0.9942 | 146.86 | 67.37 |
| SVR | 0.9968 | 108.65 | 36.03 |
| LightGBM | 0.9949 | 137.49 | 53.69 |

# 4. Conclusion

The paper was able to reproduce and replicate the methodological approach suggested by Demolli et al. to predict the long-term wind power generation through machine learning algorithms. The study not only confirmed the existing regression models through the use of meteorological data of Chicago and Detroit, but also brought with it contemporary algorithmic improvements and optimizing measures.

The most important results of this study can be formulated as follows:

Algorithmic Performance: According to the reference study, in the case of the baseline (Case 1), SVR and XGBoost showed better results compared to 0.99, having an R 2 of more than 0.99. This substantiates the fact that ensemble and support vector techniques prove quite useful in achieving the non-linear association amid wind velocity and power yield.

Modern Methodology: LightGBM with Bayesian hyperparameter optimization (Optuna) turned out to be a strong modernisation of the initial trial and error technique. LightGBM model was competitive in terms of accuracy and provided efficient training options justifying its incorporation as a strong forecasting energy tool of renewable energy.

Geographical Transferability: The important contribution of this research was the successful implementation of the "Blind Test" on the Detroit dataset (Case 3). The models which had been trained only on the Chicago climatic data could be extrapolated successfully to Detroit and this indicated that the patterns which had been learned are not site specific but instead are physically based on the wind-power relationship.

Input Sensitivity: The test proved that the addition of wind speed volatility (Standard Deviation) as an input factor (Case 1) is a much better predictor of predictability than when only mean wind speed is used (Case 2).

In conclusion, this report has shown that machine learning models, especially when optimized using modern models such as Optuna, provides a reliable and transferable solution to the problem of wind power forecasting. These tools may contribute greatly to site evaluation and energy planning in areas with high wind potentials to become sustainable development.

# 5.References

[1] H. Demolli, A. S. Dokuz, A. Ecemis, and M. Gokcek, "Wind power forecasting based on daily wind speed data using machine learning algorithms," Energy Convers. Manag., vol. 198, p. 111823, Oct. 2019.

[2] R. Bayindir, "Wind Energy Systems Lecture Notes," Department of Electrical Engineering, Gazi University. [Online].

[3] SelfishGene, "Historical Hourly Weather Data 2012-2017," Kaggle. [Online]. Available: https://www.kaggle.com/datasets/selfishgene/historical-hourly-weather-data.

[4] OpenWeatherMap, "Weather API - Current weather and forecast," OpenWeatherMap. [Online]. Available: https://openweathermap.org/api/weather-map-2.